SOFTWARE REQUIREMENTS SPECIFICATION

Renewable Energy Grid Simulator with AI-Based Demand Forecasting

An interactive microgrid simulation demonstrating forecast-driven battery scheduling for improved renewable energy utilization

Aligned with UN SDG 7 (Affordable & Clean Energy) and SDG 13 (Climate Action)

Document Version: 1.0

Date: 07 August 2026

Prepared by: Adwaith S Dev (2024418186), Gaurav Rustagi (2024519071)

Institution / Course: [Institution Name, Program]

Project Guide: [Guide Name]

# Table of Contents

(Right-click below and choose "Update Field" → "Update entire table" after opening in Word to populate page numbers.)

# 1. Introduction

## 1.1 Purpose

This document specifies the software requirements for the Renewable Energy Grid Simulator, an interactive tool that demonstrates how AI-based electricity demand forecasting improves renewable energy utilization and reduces reliance on fossil-fuel backup power in a simulated microgrid. It is intended for the project guide/evaluators reviewing the proposal, and for the developer as the authoritative reference for what is being built during the two-week implementation window.

## 1.2 Scope

The system, referred to hereafter as the “Simulator”, will:

- Generate a realistic synthetic dataset (one year, hourly resolution) of electricity demand, solar generation, and wind generation.

- Train a machine-learning model to forecast short-term electricity demand and compare its accuracy against a naive baseline forecast.

- Simulate a microgrid (solar + wind + battery + fossil backup) under two dispatch strategies — reactive (no forecasting) and forecast-driven (battery scheduled ahead of predicted demand) — and quantify the difference between them.

- Present the simulation through an interactive web dashboard where a user can adjust solar/wind/battery capacity via sliders and immediately observe the effect on renewable utilization percentage, CO₂ emissions avoided, and estimated cost saved.

Out of scope: connection to a live/real power grid or real-time sensor feeds, financial-grade cost modelling, multi-region or multi-node grid topology, and any production deployment/hosting — this is a local, offline academic demonstration tool.

## 1.3 Definitions, Acronyms and Abbreviations

- SDG: Sustainable Development Goal (United Nations).

- SoC: State of Charge — the current stored energy level of the battery, expressed as a percentage of capacity.

- MAE / RMSE: Mean Absolute Error / Root Mean Squared Error — standard regression accuracy metrics used to evaluate the forecasting model.

- Reactive Mode: Simulation mode in which battery/backup dispatch decisions are based only on the current hour’s actual demand, with no lookahead.

- Forecast-Driven Mode: Simulation mode in which battery dispatch decisions additionally use the ML model’s predicted future demand to pre-charge/discharge the battery ahead of anticipated peaks.

- Renewable Utilization %: Share of total demand met by solar + wind + battery-discharged renewable energy, over the simulation period.

- Emission Factor: Configurable constant (kg CO₂ per kWh) used to convert fossil-backup generation avoided into an estimated CO₂ emissions-avoided figure.

## 1.4 References

- United Nations Sustainable Development Goals — Goal 7 (Affordable and Clean Energy) and Goal 13 (Climate Action), sdgs.un.org

- scikit-learn documentation — scikit-learn.org

- Streamlit documentation — docs.streamlit.io

- IEEE Std 830-1998 — Recommended Practice for Software Requirements Specifications (structure reference for this document)

## 1.5 Overview

Section 2 describes the product at a high level — its context, main functions, intended users, and constraints. Section 3 lists detailed functional and non-functional requirements. Section 4 describes the system architecture and data flow. Section 5 defines representative use cases. Section 6 maps requirements to the project’s build phases and timeline. The appendix contains a glossary and the SDG alignment mapping.

# 2. Overall Description

## 2.1 Product Perspective

The Simulator is a new, self-contained, single-user desktop/local-web application. It is not a modification of an existing system and has no dependency on any live external grid, API, or paid service. It runs entirely offline once dependencies are installed, using locally generated synthetic data. It is composed of four internal modules — Data Generation, Forecasting Model, Simulation Engine, and Dashboard (UI) — described fully in Section 4.

## 2.2 Product Functions (Summary)

- Synthetic generation of a full year of hourly demand, solar, and wind data.

- Training and evaluation of an ML demand-forecasting model against a naive baseline.

- Hour-by-hour microgrid simulation with battery dispatch, under two switchable strategies.

- Interactive dashboard with live sliders and charts for exploring “what-if” capacity scenarios.

- Computation of headline impact metrics: renewable %, CO₂ avoided, cost saved.

## 2.3 User Characteristics

- Primary user — Evaluator / Project Guide: Views a live demo, operates sliders and the mode toggle, and interprets the displayed charts and metrics. No programming or data-science background assumed; all controls must be self-explanatory.

- Secondary user — Developer: Runs the data-generation and model-training scripts, inspects accuracy metrics, and maintains the codebase. Assumed to have working knowledge of Python.

## 2.4 Constraints

- Two-week solo development timeline — requirements are scoped to be realistically deliverable within this window (see Section 6).

- Must run fully offline during the demo, so the presentation is not dependent on network/venue Wi-Fi reliability.

- Built with Python, scikit-learn, and Streamlit specifically, per the approved project pitch.

- Uses synthetic, not live, data — chosen deliberately so demo behaviour is reliable and repeatable regardless of internet access or real-world data availability on presentation day.

## 2.5 Assumptions and Dependencies

- A statistically realistic synthetic dataset is an acceptable substitute for real utility data for the purpose of this academic demonstration; absolute forecast accuracy is not the goal — the relative improvement over the naive baseline, and its downstream effect on renewable utilization, is.

- The demonstration machine has Python 3.9+ and the required packages (pandas, NumPy, scikit-learn, Streamlit, Matplotlib/Plotly) installed prior to the demo.

- A single, generic CO₂ emission factor and a single, generic electricity cost rate are sufficient for the illustrative cost/CO₂ figures — these are explicitly presented as estimates, not audited financial figures.

- Should real public grid/weather datasets later be substituted for synthetic data (an open question for the guide, per the pitch brief), this SRS's functional requirements are expected to remain valid; only the Data Generation module's implementation would change.

# 3. Specific Requirements

## 3.1 Functional Requirements

### 3.1.1 Data Generation Module (FR-DG)

| ID | Requirement Description | Priority |
| --- | --- | --- |
| FR-DG-1 | The system shall generate a synthetic hourly electricity demand dataset covering one full year (8,760 records), exhibiting a daily double-peak pattern (morning and evening), a weekday/weekend distinction, seasonal variation, and randomized noise. | High |
| FR-DG-2 | The system shall generate a synthetic hourly solar generation profile bounded by daylight hours, with seasonal amplitude variation and randomized cloud-cover noise. | High |
| FR-DG-3 | The system shall generate a synthetic hourly wind generation profile using a semi-random distribution that is largely independent of time of day. | High |
| FR-DG-4 | The system shall persist all generated datasets to local files (CSV) so they can be reused without regeneration on every run. | Medium |
| FR-DG-5 | The system shall provide summary plots of the generated dataset so its realism can be visually verified before use in later modules. | Medium |

### 3.1.2 Forecasting Model Module (FR-FM)

| ID | Requirement Description | Priority |
| --- | --- | --- |
| FR-FM-1 | The system shall engineer time-based features (hour of day, day of week, month, lag values, rolling averages) from the demand dataset for use in model training. | High |
| FR-FM-2 | The system shall train a scikit-learn regression model to forecast electricity demand for a short future horizon (next 1–24 hours). | High |
| FR-FM-3 | The system shall implement a naive baseline forecast (same hour, previous day) to serve as a comparison benchmark. | High |
| FR-FM-4 | The system shall evaluate and report forecast accuracy (MAE and RMSE) for both the trained model and the naive baseline on a held-out test set. | High |
| FR-FM-5 | The system shall serialize the trained model to disk so the Simulation Engine can load it without retraining on every run. | Medium |

### 3.1.3 Simulation Engine Module (FR-SE)

| ID | Requirement Description | Priority |
| --- | --- | --- |
| FR-SE-1 | The system shall simulate hourly renewable generation (solar + wind) scaled by user-specified capacity parameters. | High |
| FR-SE-2 | The system shall simulate battery state of charge across the simulation period, applying charge/discharge logic each simulated hour. | High |
| FR-SE-3 | The system shall support two dispatch modes: Reactive Mode (decisions based only on current-hour demand) and Forecast-Driven Mode (decisions additionally informed by the FR-FM forecasting model). | High |
| FR-SE-4 | The system shall compute the fossil-fuel backup generation required to cover any shortfall between available supply (renewable plus battery) and actual demand, for each mode. | High |
| FR-SE-5 | The system shall compute, per simulation run: renewable utilization percentage, estimated CO₂ emissions avoided, and estimated cost saved relative to a fossil-only baseline. | High |
| FR-SE-6 | The system shall allow a simulation to be re-run with modified capacity/battery parameters without restarting the application. | Medium |

### 3.1.4 Dashboard / UI Module (FR-UI)

| ID | Requirement Description | Priority |
| --- | --- | --- |
| FR-UI-1 | The system shall provide a web-based interactive dashboard (built with Streamlit), accessible via a local browser session. | High |
| FR-UI-2 | The dashboard shall provide interactive controls (sliders) for solar capacity, wind capacity, and battery capacity/power rating. | High |
| FR-UI-3 | The dashboard shall provide a control to switch between Reactive Mode and Forecast-Driven Mode. | High |
| FR-UI-4 | The dashboard shall display a time-series chart comparing demand against total supply (renewable + battery + fossil backup) for the active simulation run. | High |
| FR-UI-5 | The dashboard shall display a chart of battery state of charge over the simulation period. | Medium |
| FR-UI-6 | The dashboard shall display summary stat tiles for renewable utilization %, CO₂ avoided, and cost saved, updating immediately when any control changes. | High |
| FR-UI-7 | The dashboard shall visually reference SDG 7 and SDG 13 alignment in its framing/labelling of results. | Low |

## 3.2 Non-Functional Requirements

| Category | Requirement | Target / Metric |
| --- | --- | --- |
| Performance | The dashboard shall reflect a slider or mode change in its charts and metrics without a noticeable, unexplained delay. | ≤ 2 seconds |
| Usability | A first-time user shall be able to operate every control using only on-screen labels, without a manual. | No training required |
| Reliability | The system shall not crash or produce undefined/NaN output for any in-range control value, including boundary values. | 0 unhandled errors |
| Portability | The system shall run on any machine with a supported Python version installed, without requiring internet access at runtime. | Python 3.9+, offline |
| Maintainability | Data generation, forecasting, simulation, and UI logic shall be implemented as separable modules with clear function-level interfaces. | 4 distinct modules |
| Security / Privacy | No user data is collected, stored, or transmitted; the tool operates on synthetic data only. | Not applicable / by design |

## 3.3 External Interface Requirements

| Interface | Description |
| --- | --- |
| User Interface | Web-based dashboard rendered by Streamlit and viewed in the user’s default browser at a local address (localhost). All input is via sliders, dropdowns, and buttons — no command-line interaction required at demo time. |
| Hardware Interface | None. Runs on standard consumer laptop/desktop hardware; no GPU, sensors, or specialized peripherals required. |
| Software Interface | Python 3.9+; pandas and NumPy for data handling; scikit-learn for model training/inference; Streamlit for the UI layer; Matplotlib and/or Plotly for charting. |
| Communication Interface | None required for core operation (fully offline). Internet access would only be needed if a public real-world dataset is substituted for synthetic data at a later stage. |

# 4. System Architecture / Data Flow

The system is organized as a linear pipeline of four modules, each consuming the previous module’s output. This separation (Section 3.2, Maintainability) allows any single stage — e.g. swapping synthetic data for a real dataset — to be replaced without changes to the others.

| Stage | Module | Input | Output |
| --- | --- | --- | --- |
| 1 | Data Generation | Configuration parameters (year length, seasonal/noise settings) | demand.csv, solar.csv, wind.csv (hourly, 1 year) |
| 2 | Forecasting Model | demand.csv (engineered into time-based features) | Trained model file (.pkl) + accuracy report (MAE/RMSE vs. baseline) |
| 3 | Simulation Engine | solar.csv, wind.csv, demand.csv, trained model, user capacity parameters | Per-hour supply/battery/backup series + summary metrics (renewable %, CO₂ avoided, cost saved) |
| 4 | Dashboard (UI) | Simulation Engine outputs, live user control values | Rendered charts and stat tiles in the browser |

At runtime, moving a dashboard slider (Stage 4) triggers Stage 3 to re-run the simulation with the new parameters; Stages 1 and 2 are pre-computed once and reused across every simulation run, which is what keeps the live-demo response time within the Section 3.2 performance target.

# 5. Use Cases

| ID | Name | Actor | Description |
| --- | --- | --- | --- |
| UC-1 | Adjust capacity scenario | Evaluator | User moves the solar / wind / battery sliders; the simulation re-runs and the charts and impact metrics update immediately to reflect the new scenario. |
| UC-2 | Compare dispatch strategies | Evaluator | User toggles between Reactive Mode and Forecast-Driven Mode for the same capacity scenario and observes the change in renewable %, CO₂ avoided, and cost saved — the core proof point of the pitch. |
| UC-3 | Review forecast accuracy | Developer | Developer inspects the MAE/RMSE report comparing the trained model against the naive baseline, to confirm the forecasting component is genuinely adding value before it is relied on in Forecast-Driven Mode. |
| UC-4 | Regenerate synthetic dataset | Developer | Developer re-runs the Data Generation module (optionally with adjusted noise/seasonality settings) to produce a fresh year of demand/solar/wind data for testing. |

# 6. Traceability to Project Phases

This maps each requirement group above to the phase of the two-week build plan in which it is delivered, so progress against this SRS can be tracked directly.

| Phase | Timeframe | Requirements Covered | Key Deliverable |
| --- | --- | --- | --- |
| 0 | Day 1 | Project setup; scope lock for all FR groups | Running empty dashboard shell |
| 1 | Day 1–3 | FR-DG-1 to FR-DG-5 | One year of synthetic demand/solar/wind data |
| 2 | Day 3–6 | FR-FM-1 to FR-FM-5 | Trained forecasting model beating the naive baseline |
| 3 | Day 6–9 | FR-SE-1 to FR-SE-6 | Working simulation engine, both dispatch modes |
| 4 | Day 9–12 | FR-UI-1 to FR-UI-7 | Full interactive dashboard |
| 5 | Day 12–14 | NFR-Performance, NFR-Reliability (Section 3.2); demo rehearsal | Stress-tested, demo-ready build + report/slides |

# 7. Tools and Technologies Used

All tools below are free/open-source and run entirely on local hardware — no paid service, API key, or cloud subscription is required to build, run, or demo the Simulator.

| Category | Tool / Technology | Purpose in This Project | Cost / License |
| --- | --- | --- | --- |
| Language | Python 3.9+ | Core implementation language for all four modules. | Free, open-source |
| Data handling | pandas, NumPy | Synthetic dataset generation, feature engineering, time-series manipulation. | Free, open-source |
| Machine learning | scikit-learn | Training and evaluating the demand-forecasting model (Random Forest / Gradient Boosting) and the naive baseline. | Free, open-source |
| Model persistence | pickle / joblib | Saving the trained model so the Simulation Engine can load it without retraining. | Built into Python / scikit-learn |
| Visualization | Matplotlib and/or Plotly | Dataset sanity-check plots (Phase 1) and in-dashboard charts (demand vs. supply, battery SoC). | Free, open-source |
| Dashboard / UI | Streamlit | Renders the interactive web dashboard — sliders, mode toggle, live charts and stat tiles. | Free, open-source |
| Development environment | VS Code | Code editing and debugging. | Free |
| Version control | Git (+ GitHub, optional) | Source control and backup of the codebase during the two-week build. | Free |
| Documentation | Word / Markdown | This SRS, the project report, and presentation notes. | Free / existing license |

# 8. Project Budget

Because the Simulator is built entirely from free, open-source tools (Section 7), runs on existing personal hardware, and uses synthetically generated data rather than a purchased dataset or paid API, the monetary budget is effectively zero. The real constraint on this project is developer time, not money — the effort budget below breaks that down by phase, matching the phased plan in Section 6.

## 8.1 Monetary Budget

| Item | Estimated Cost | Notes |
| --- | --- | --- |
| Software & libraries | ₹0 | Python, pandas, NumPy, scikit-learn, Streamlit, Matplotlib/Plotly — all free and open-source. |
| Hardware | ₹0 | Runs on existing personal laptop; no GPU or specialized equipment required (Section 2.4). |
| Dataset | ₹0 | Synthetically generated in-project (Phase 1); no purchased or licensed dataset used. |
| Hosting / deployment | ₹0 | Demo runs fully offline on localhost; no cloud hosting needed (Section 2.4). |
| Miscellaneous (printing, etc.) | ₹0 – ₹500 | Optional — printed report/handout copies only, if required by the guide. |
| Total estimated monetary cost | ₹0 – ₹500 | Approx. US$0–6. No budget approval / funding request is needed for this project. |

## 8.2 Effort Budget (Developer Time)

| Phase | Activity | Est. Effort (hrs) | Cumulative (hrs) |
| --- | --- | --- | --- |
| 0 | Setup & scope lock | 3 | 3 |
| 1 | Synthetic data generation | 8 | 11 |
| 2 | Forecasting model | 10 | 21 |
| 3 | Grid simulation engine | 10 | 31 |
| 4 | Streamlit dashboard | 10 | 41 |
| 5 | Polish, stress-test, demo prep | 8 | 49 |

≈ 49 person-hours over 14 days (roughly 3.5 hrs/day average) for a solo build — consistent with the two-week timeline proposed in the pitch.

# Appendix A — SDG Alignment

| SDG | How the Simulator Demonstrates It |
| --- | --- |
| SDG 7 — Affordable and Clean Energy | Shows, with live numbers, how forecast-driven scheduling raises the share of demand met by solar/wind/battery rather than fossil backup — i.e., more effective use of the same clean-energy capacity. |
| SDG 13 — Climate Action | Translates that improved renewable utilization directly into an estimated CO₂ emissions-avoided figure, making the climate impact of forecasting-driven grid management tangible and interactive rather than abstract. |

# Appendix B — Open Questions for the Project Guide

- Preference for real public datasets over synthetic data — adds credibility, but risks demo-day reliability if network/data access is unavailable.

- Whether SDG alignment should be a dedicated section in the final report/demo, or woven throughout the presentation.

- Whether the current scope is right-sized for the two-week solo timeline, too ambitious, or should be trimmed (see Section 2.4 constraints and the fallback in Section 6).